Installation of Wastewater Treatment Systems Homework

PROBLEM 1:  Evaluating a Topographic Map

1.1	How many drainage paths flow in a Southwest direction on this site?

1.2	What is the approximate average slope of the site along the A-A line?

PROBLEM 2:  Layout of Curtain Drain and Material Estimation

2.1  What is the approximate flow line elevation at the beginning of the curtain drain pipe, if the pipe bottom is buried 3.5 feet deep?

2.2.  What is the approximate flow line elevation at the exit of the curtain drain assuming the bottom of the pipe exit is 1 foot above the bottom of the surface ditch?

2.3.  Assuming the above elevations and the upslope drain line is placed on contour at a constant elevation, what would be the slope of the curtain drain?

  - Assuming the above trenches have dimensions of 3 foot width, 1 foot depth and 75 foot length, calculate the volume of washed rock media needed to construct the trenches?

How many leaching chambers would be needed to construct the trenches, assuming a chamber length of 6.25 feet?

How many 4.5 foot long chambers?

PROBLEM 3:  Pump Capacity Calculation

The system needs a pump delivery rate of 50 gpm for proper operation.  The tank inside dimensions are 60”width x 60” length and an operating depth of 50”.

  - What is the tank capacity in gallons?

_   Length (ft) x   _      Width (ft)  x          Depth (ft)  x 7.5 gal/cubic ft =            gallons

3.2	What is the tank capacity in gallons per inch?

tank capacity in gals. ÷ operating depth of tank in inches       OR

[          Length (ft) x            Width (ft)  x  7.5]  ÷ 12 =          gallons per inch

3.3	What is the pump capacity in gallons per minute?  (Pump on measurement of 42”, Pump off measurement of 56”, pump run time of 3 minutes)

Difference in depth x gallons per inch ÷ time = pump capacity in gpm

(        in. (OFF) -         in. (ON)) x          gal / inch ÷         minutes  =                           gpm

PROBLEM 4:  Elevation Calculations for a Septic Tank

Rod reading at flow line of outlet pipe = 8.80 feet

Piping to tank:  16 feet of 4 inch pipe.

Benchmark elevation = 100 feet

Rod reading for backsight to Benchmark = 9.75 feet

Rod reading at point A = 6.84 feet

Tank wall and bottom thickness of 3 inches

4.1	What is the rod reading for determining the bottom of the excavation?

4.2	What is the elevation of the bottom of the excavation?

  - What is the depth of cover over the top of the tank?

PROBLEM 5:  Determining Excavation Elevation

Assume a minimum 1/8 inch per foot slope on pipe

Plumbing Stub Out elevation (flow line) = 93.00 feet

Piping to tank:  24 feet of 4 inch pipe

Benchmark elevation = 98 feet

HI = 101.25 feet.  [Rod Reading = 3.25 (HI – BM = RR)]

Tank wall and bottom thickness of 3 inches

Tank outside dimensions: 5’ width x 7’ length x 68” height

Ground surface elevation at tank excavation: 94.21 feet

5.1	What is the elevation at the bottom of the excavation?

5.2	What is the rod reading for determining the bottom of the excavation?

PROBLEM 6:  Capacity calculations for a cylindrical tank

A septic tank being installed has the configuration shown below:

INSIDE DIAMETER of the tank = 8’- 3”

Operating depth (depth from the invert of the OUTLET pipe to the interior floor of the tank) = 5’- 4”.

Remember to first convert inches to feet.

6.1  What is the volume of the tank?

π = 3.14 (a constant, does not change)  R = half the diameter  Area of circle =  π R2

First find the area of the tank circle; then calculate the volume of the tank in cubic feet, and then calculate capacity in gallons.

  - What is the tank capacity in gallons per inch?

Installer Training Program Homework Questions

1.   	T 	F 	The use of standardized installation checklists can promote clear communication between industry professionals and their clients.

2.	T	F	A professional installer should possess basic knowledge of the functions of the wastewater system treatment train components.

3.	T	F	The installer should view the design as general guidance on placement of onsite wastewater treatment system components on the site.

4.	T	F	Installation of wastewater treatment system components at an incorrect elevation can lead to a malfunction of the treatment system.

5.	T	F	The equipment a practitioner owns is the main factor determining their business model for working in the wastewater industry.

6.	T	F	An installer should check the local roles and responsibilities because local inspectors and ordinances may define installer responsibilities in their rules or guidance documents.

7.	T	F	A customers’ expectations regarding your job performance is shaped by their past experiences when working with the industry.

8.	T	F	A successful bid and contract can always be negotiated without a site visit.

9.	T	F	Most homeowners do not care how the system looks, they just want it to work.

10.	T		F	A utility locate (one call) is ordered and obtained before starting construction.

11.	T		F	The development of a biomat is a critical process for uniform distribution in a gravity distribution trench.

12.	T		F	An aerobic soil condition below trenches is critical for pathogen removal from wastewater.

13.	T	F	The long term acceptance rate of wastewater by a clay soil is greater than a sand soil.

14.	T	F	The ability for water to move down slope on a given site can be reduced by construction of a driveway (compaction) down slope of the soil treatment area.

15. 	T	F	A soil treatment area with the greatest dimension along the contour will have a smaller or lesser contour loading rate than a soil treatment area with the greatest dimension running up and down the slope.

16.	T	F	A large dozer is the best equipment choice for removing vegetation from the soil treatment area.

17.	T	F	The soil treatment area should be roped-off and protected from access during construction of a house or commercial building.

18.	T	F	The soil treatment area is the best place for the concrete crew to wash out the concrete trucks.

19.	T	F	One-quarter inch (¼”) of fine particles or less in a jar test is an appropriate field verification measurement of clean mound/media filter sand.

20.	T	F	Wheeled equipment can be driven over a roughened soil treatment area as long as the soil is not wet.

21.	T	F	SDR and Schedule mean the same thing in reference to pipe.

22.	T	F	Both primer and glue should be used when solvent welding PVC pipe.

23. 	T	F	The OSHA soil classification system and the USDA soil classification system use the same approach to classification of soil.

24.	T	F	A competent person should be present on the site if an excavation greater than four feet in depth is required, and somebody will enter the excavation.

25.	T	F	A soil having water present in the excavation would be classified by OSHA as a type A soil.

26.	T  	F	A soil that freely flows from the excavation bucket will probably be classified as an OSHA type C soil.

27.	T	F 	A soil that falls from the excavation bucket in free standing clumps will be classified as an OSHA type A soil.

28.	T	F	A natural cohesive soil located on an undisturbed site with a compressive strength greater than 1.5 tons per square foot and no water in the excavation would be classified as an OSHA type A soil.

29.	T	F	A granular soil located on an undisturbed site, with no water present in the excavation and a compressive strength less than 0.5 tons per square foot would be classified as an OSHA type C soil.

30.	On a job site, the person in charge of safety is the (choose the best answer):

a. Job supervisor

b. Competent person

c. Company owner

d. Everyone