Form 5-2 Operational Checklist:  Septic, trash and processing tanks (STPT)

Service provided on:   Date:               	Time:                        		Reference #:

Service provided by:   Company: 				     	  	Employee:

Date of last service: 		 		By:   You    Other:

Date of last inspection: _______________________________

1.     Type:

 Septic tank                 Trash tank

NOTES

 Processing tank          Pump vault present

2.     Conditions at the tank

2.   Acceptable

 Unacceptable

4.  ð Acceptable

ð Unacceptable

5.  ð Acceptable

ð Unacceptable

6.  ð Acceptable

ð Unacceptable

a.	Evaluate presence of odor within 10 ft of perimeter of system:

ð None     ð Mild     ð Strong      ð Chemical     ð Sour

b.	Source of odor, if present: ________________________________

3.	Tank description

a.	Material:       Concrete          Fiberglass          Plastic

b.	Capacity:	                        gal

c.	Compartmented.	Yes  	No

d.	Capacities for compartmented system: 1)______ gal   2) ______ gal

4.	Tank access

a.	Access location:               Inlet              Outlet              Center

b.	Located at grade.	Yes	No

c.	If ‘No’, how deep is lid buried.

d.	Risers on tank.	Yes	No

e.	Evidence of infiltration in risers.	Yes	No

f.	Lids securely fastened.	Yes	No

g.	Lid in operable condition.	Yes	No

5.	Alarm(s)

a.	Alarm(s) present.	Yes	No

b.	Audio alarm operational.	N.A. 	Yes	No

c.	Visual alarm operational.	N.A. 	Yes	No

d.	Remote telemetry operational.	N.A. 	Yes	No

e.	Electronic monitoring operational.	N.A. 	Yes	No

6.	Current tank operating conditions

a.    Liquid level relative to outlet:                         			in

 At          Above        Below

b.	Maximum liquid level of tank (invert of inlet pipe):		in

c.	Height at which alarm is activated as measured

from invert of inlet:				in

d.	Evidence liquid level has been higher.	Yes	No

e.	Evidence liquid level dropped without pumping.	Yes	No

f.	Evidence of continuous inflow.	Yes	No

g.     Date of last pumpout:_____________________________________

h.	Presence of flocculant in clear zone.                     Yes        No

i.	Evaluation of layers in tank:

| Compartment Number | Scum (in) |  | Clear Zone (in) |  | Sludge (in) |  | Odor | Other |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Depth | Color* | Depth | Color | Depth | Color |  |  |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |

*Color Choices: 	ð Clear  	ð Flocced  ð Milky      ð Muddy  ð Grainy

ð Black  ð Brown   ð Mustard   ð Gray      ð White

7.	Tank pumping recommended.	Yes	No

Reference #:

8.    ð Acceptable

ð Unacceptable

9.    ð Acceptable

ð Unacceptable

8.	Baffles currently structurally sound.	Yes	No							a.	Inlet baffle in place.	Yes	No

b.	Outlet baffle in place.	Yes	No

c.	Compartment baffle in place.          N.A. ______	Yes	No

d.	Effluent screen.	Yes	No

Manufacturer:	Model:

e.	Is screen accessible from ground surface.	Yes	No

f.	If screened, percent plugged:		            %

g.	Was screen cleaned.	Yes	No

9.	Tank structural condition (evaluate if tank pumped):	N.A.

a.	Appears to be watertight (no visual leaks).	Yes	No

b.	Rebar exposed.	Yes	No

c.	Corrosion present.	Yes	No

d.	Spalling present.	Yes	No

e.	Cracks present.	Yes	No

f.	Root intrusion.	Yes	No

g.	Deflection noted.	N.A.	Yes	No

10.	Contractor responsible for pumping:

a.	Gal removed:_______________        Date:____________________

11.	Lab samples collected for monitoring.	Yes	No

Types of analysis: