Form 6-2 Operational Checklist:  Pump:  Demand-dosed system (PDD) (Including siphons)

Service provided on:   Date:               	Time:                        		Reference #:

Service provided by:   Company: 				     	  	Employee:

Date of last service: 		 		By:   You    Other:

Date of last inspection: _______________________________

NOTES

System type:    ð Pump   ð Siphon

1.   Acceptable

 Unacceptable

2.  ð Acceptable

ð Unacceptable

3.  ð Acceptable

ð Unacceptable

1.	Controls

a.	Type:           Piggy back               Control panel

b.	Controls operating properly.	        		Yes _____  No______

c.	Is enclosure watertight.                                    		Yes ______No______

d. 	Alarm test switch working properly.                 		Yes ______No______

e.	At time of inspection, control switch (HAND-OFF-AUTO) was set at:

“Hand/Manual”

“Auto”

“Off”

f.	Electrical meter readings:

|  |  | Reading (this) | Reading (last) | Difference | N.A. |
| --- | --- | --- | --- | --- | --- |
| i) | ETM |  |  | min |  |
| ii) | Cycles/events |  |  | Events (NC) |  |

Calculate cycles/day: _____ [NC] / [Days] = _______ [CPD]

g.	Telemetry operational.                         	N.A.: 		 Yes ______No_____

Type:

2. 	Pump/Siphon

a.    Siphon operating properly.		N.A.: 	Yes ______No_____

b.	Pump operating properly.                                           	Yes ______No_____

c.	Type of pump:                         	 ð  Multi-stage  	ð  Single-stage

d.	Amps measured:                                               	 _____________amps

e.	Voltage measured:	                	 _____________volts

f.     Pump turns on/turns off.	                 	Yes _____No______

3.	Water level sensors

a. 	Type of water level sensor:  	ð  Floats        		ð  Pressure transducers

ð  Ultrasonic   	ð  Other:

b. 	Pump floats/sensors functioning properly.                     Yes _____No______

c. 	Alarm float/sensor operating both audible and visible. Yes _____ No______

4.	Sensor settings:

| Sensor Number* | Function | Operational | Set At** |  | Secured |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Inches | Datum |  |
| 1 |  | Yes___ No___ |  |  | Yes___ No___ |
| 2 |  | Yes___ No___ |  |  | Yes___ No___ |
| 3 |  | Yes___ No___ |  |  | Yes___ No___ |
| 4 |  | Yes___ No___ |  |  | Yes___ No___ |
| 5 |  | Yes___ No___ |  |  | Yes___ No___ |

*(Designate starting from bottom of tank)

** (Measurements are taken from a fixed point (“Datum”) near the surface or bottom of float tree in                 	     inches)

5.	Dose volume (DV)

a. 	Pump Off – Pump On = ________in pumped (dose)

b. 	GPI: _______      (Form 6.1 – Item 3.e)

_______dose (in) x _______________GPI =____________ DV(gal)

Reference #:

6.	Pump delivery rate (PDR)

a.	Dose volume (from Item 5):					 __________ gal

b. 	Verified pump run time “On”:				 	 __________ min

_____________ gal pumped ÷ ___________ min  = __________________ GPM

7.	Total gallons

a.	Method to activate pump:        ð Water added              ð Lifted float

b.	Total gallons (from elapsed time meter)

[_____(PTR) - _____(LTR)] x  _____(GPM) = _______Total Gal

OR Total gallons (from event/cycle counter)

[_____(PCR) - _____(LCR)] x  _____(DV)    = _______Total Gal

8.    Gallons per day (GPD)

a.	_____Total gal  _____No. of days = ________Gal/day (GPD)

CPD: Cycles per day

DV:  dose volume

ETM: Elapsed time meter

GPI: gallons per inch

GPM:  gallons per minute

GPD: gallons per day

HAND-OFF-AUTO:  Hand-Off-Auto Switch

LCR: last cycle reading

LTR: last time reading

PCR:  present cycle reading

PDR: pump delivery rate

PTR:  present time reading